Begriffe:

Uplink: Nächster schritt nach oben

Endgeräte > Access-Switches > Core-Switches

Accessport: zu Endgerät (PC)

Trunkport: zu Switch oder Router

Tagged Frames: Dot1Q-Frames mit VLAN-ID.

Untagged Frames: Frames ohne VLAN-ID, verwendet an Access-Ports.

Kupferports (RJ45)

SFP- Ports (Small Form-factor Pluggable)

WEP: Wired Equivalent Privacy

WPA : Wi-Fi Protected Access

MIB: Management Information Base

OID: Object identifier (MIB Zahl)

GRE : Generic Routing Encapsulation

MPLS: Multiprotocol Labelswitching

MTU : Maximum Transmission Unit

SSID : Service Set Identifier

SNMP-Agent:  Software auf einem Netzwerkgerät (Router, Switch) läuft.

SNMP-Trap: Alarmmeldung, die vom SNMP-Agenten an den SNMP-Manager

SNMP-Community: einfacher Klartext-String, der als Passwort dient

- public für lesenden (GET).

- private für schreibenden (SET).

nano /etc/snmpd.conf

SNMP im FCAPS Modell

ein älteres Modell das den Betrieb eines Netzwerks anhand von fünf Bereichen beschreibt:

- Fault management

- Configuration management

- Accounting management

- Performance management

- Security management

Fault Management ist das Erkennen, Isolieren, Beheben und Protokollieren von im Netz aufgetretenen Fehlern. Performance Management sammelt und analysiert Leistungsdaten und will das Netz für die Zukunft vorzubereiten

Default Port befehle Cisco CLI im: conf t

TIPP: bei zweifel tabulator drücken

interface gi0/1

encapsulation dot1q (Für router )

description xxxx

switchport mode access / trunk

switchport access vlan 5

switchport trunk allowed vlan

switchport native vlan

sh  ip int brief

sh vlan br

| VPN-Protokoll | IPsec | L2TP | PPTP | MPLS |
| --- | --- | --- | --- | --- |
| OSI-Schicht | 3 | 2 | 2 | 2 |
| Standard | Ja | Ja | Nein | Ja |
| Paketauthentisierung | Ja | Nein | Nein | Nein |
| Datenverschlüsselung | Ja | Nein | Ja | Nein |
| IP-Tunneling | Ja | Ja | Ja | Ja |
| Hauptanwendung | End-to-End | Provider | End-to-End | Netzbetreiber |

5 GHz

Mit rund 380 MHz verfügbarer Bandbreite bietet es erheblich mehr Raum für Übertragungen. Die Kanäle sind überlappunsfrei.

2.4 GHz

Die Bandbreite ist ca. 80 MHz. Die Kanäle überschneiden sind.

5. Tunnel Between Two Routers Through the Internet

Potential Issues:

- Tunnel down due to incorrect source/destination.

- Routing issues between private networks.

- Public IP addresses misconfigured.

Troubleshooting Steps:

Verify Tunnel Interface:

show ip interface brief | include Tunnel

Tunnel is up/up.

Check Tunnel Source/Destination:

show running-config interface Tunnel0

correct public IPs configured as tunnel src &dest.

Verify Static Routes:

show ip route

3. Router Default-Gateway for Multiple VLANs

Potential Issues:

- Subinterfaces not configured or misconfigured.

- Incorrect IP address or encapsulation on subinterfaces.

- Default gateway not set on the switch.

Troubleshooting Steps:

Verify Router Subinterfaces:

show ip interface brief

Check Default Gateway on Switch:

show running-config | include ip default-gateway

correct ip default-gateway.

Check Trunk Port Connection:

show cdp neighbors

2. Access and Trunk Ports

Potential Issues:

- Access port not assigned to the correct VLAN.

- Trunk port not set or encapsulation missing.

Troubleshooting Steps:

Check Access Port:

show vlan brief

Check Trunk Port:

show interfaces trunk

Verify Spanning Tree Status:

show spanning-tree

4. VPN Configuration on Cisco Router (GRE)

Potential Issues:

- Tunnel interface is down.

- Incorrect tunnel source/destination IP.

- Missing static routes for remote networks.

Troubleshooting Steps:

Check Tunnel Status:

show ip interface brief | include Tunnel

Ensure the tunnel is up/up.

Verify Tunnel Configuration:

show running-config interface Tunnel0

correct source, destination, and IP address.

Check Routing:

show ip route

route to the remote network exists via the tunnel.

Ping Across the Tunnel

1. VLAN (802.1Q) Configuration on Switch and Router

Potential Issues:

- VLAN not configured correctly.

- Access ports assigned to the wrong VLAN.

- Trunk port misconfigured (missing encapsulation).

Troubleshooting Steps:

Verify VLAN Configuration on Switch:

show vlan brief

Check Access Port Configuration:

show running-config interface <INTERFACE_ID>

Check Trunk Port Configuration:

show interfaces trunk

Verify Router Subinterfaces:

show running-config interface < INTERFACE>

Test Connectivity:

Ping the router's VLAN gateway